Following is the full list of revisions. Every revision was made by Giselle D. Oldest revisions are at the bottom.

Changed frontend pathing to be relative.

----

Made changes to make the project build. Unfortunately, there was a file missing that was never committed. That one is the libarm_cortexM4l_math.a. The project will not build without this. Also, the code for the nandflash was removed. If we ever need a nandflash, then we can recover the files from SVN. Additionally, a bug was fixed for a temp conversion function where the semaphore was released too early and wasn't actually providing protection.

----

Comment adding clarification.

----

This commit is just adding a comment to http server

----

Critical bug was fixed. The IP address would not update correctly on the web server and would cause permanent fault of the system. This was caused by the max size of a network mtu being too small. It was set to 700. Apparently the http server expects a POST to come in one packet so if the POST is too big then it causes bad behavior. It was changed to 1000 and the other changes increasing the TCP window and buffer sizes were set back to all 4s since the update done in the last rev didn't really improve performance significantly and there were issues with too much RAM consumption.

----

Modbus task updated to allow the client to write to the setpoint register and the temp system register and actually have it update the system. This version of the code was tested overnight and appears to be stable.

----

The modbus task was refactored significantly to improve safety. The network settings in FreeRTOSIPConfig.h were modified to optimize socket send and receive. A preliminary test showed greater reliability with the modbus task, and the FTP server is able to receive files with less errors. The display driver and setpoint handler were modified moderately, mostly comment updates, white space changes, and minor organizational changes. Once small change was made to index.html, the heading on the home page was altered to be more relevant.

----

I decided to overhaul the display driver since it was getting unwieldy and I feared it would be difficult to maintain in the future. Previously, the display driver managed the temperature system, and the other modules did not have knowledge of what temperature system was being used. I decided to give this responsibility to the temperature reading processor since it seems like a more natural role. In the previous commit, the display driver was also managing two modbus holding registers, that task was given to the setpoint handler since that was also the natural place for that to be. The display handler is now smaller and more focused. The temp reading processor is handling more stuff. Also, in the previous commit, a merge conflict had broken modbus. This commit fixes it. Another thing is that the hard fault handler has been improved to allow easier debugging of hard faults. A minor change was made to the displayHandler in the website front end to allow compatibility with the new firmware.

----

Update to fix modbus packet drop issue. The temperature is no longer an input register, it is now a holding register. Also the setpoint and temp system were added as holding registers.

----

Some adjustments made to improve the http server and ftp upload. Modbus task priority was updated.

----

Server was updated to use fetch api to get the current temperature from SAM. The backend was updated to support a get request and return the temperature. Temp reading main was editted to stay alive along with the server so that it can provide temp readings. The heap size variable was pushed to its maximum.

----

IPV6 UI has been removed from the website since it wont be supported in the initial release. Some changes were made to the ftp and http server send functions. I made it so that if freertos_send doesn't send all the data its told to, then it will keep trying until it succeeds. I also increased the tcp windows for both servers to help improve performance.

----

This branch includes a start for making SAM ipv6 compatible. It currently is not fully functional. I believe fundamental changes to the underlying networkinterface driver are needed to achieve this milestone

----

----

Change was made that appears to fix the problem seen previously where outward network traffic would fail. Before I had fixed this by modifying an operating system file. This was only a bandaid though. The fix here appears to have fixed the problem entirely by addressing the cause and has even boosted the speed of the network. It may not be the last update though if any problems are later discovered.

----

Critical update was made to HexModule.c and the setpoint handler. There was an issue with the applications use of globals and a shared temperature conversion function. A thread collision issue was causing the application to crash due to stack corruption. This has probably been an issue for awhile and simply wasn't noticed until now. The globals in hex module have been removed and semaphore protection has been added to the toFahrenheit function since it is shared between modules.

----

Huge website update! UI has been changed to be more snazzy, the reboot feature was edited a bit, a setpoint handler was added, also the mac address wasn't being loaded at initialization and that has been addressed.

----

Refactored the display driver to clean up some clumsiness. Added new comments to both the display driver and the setpoint handler.

----

Main accomplishments today were getting the reboot feature completely working and getting most the work done for updating the setpoint with the web server. Some changes were made to the server code, mostly just small improvements. For example, the way the setpoint was being saved wasn't optimal, and boundary checking wasn't very safe for the display hander. I made it so that no matter what temperature system is used, the display will never overflow or try to publish a setpoint that is out of bounds.

----

Another update, lots of the backend capability of the server has been added. IP address and other network settings are set at initialization. The http post command is in the process of being made more generic. The goal is to have POST work for a reboot command and possibly for a setpoint update command. Some changes were made to slow down the network loading speed a little bit to improve reliability. Also, lots of unnecessary project files were removed.

----

The main update here is to the front end portion of the web server. The network settings page has been changed substantially to better suit the application and to apply proper error checking. A small update was made to the backend to change the behavior of how the settings.json file is edited. Before, it was appended to. Now it is recreated whenever the network settings form is posted.

----

Fixed a bug that was mysterious for awhile. The network interface would occasionally crash on a config assert. I found out that the problem was that occasionally the network would fail to get a buffer because the semaphore wasn't available within the given timeout. I added a while loop to force the network driver to block until it gets a buffer instead of just killing the whole program. I also optimized the speed of network transmission, so now operations like loading the webpage happen about 5 times faster. I also changed the RAM back to a level suitable for a 160kB ram size instead of the 128kB. The device runs much more reliably with this amount of ram.

----

Multifile update to improve safety concerning the deleteNonWebTasks function. I was concerned that the interprocess communication could rarely cause an unrecoverable exception because of the way the threads were shut down. I guarded queuereceive and queuesend calls with null checks and I moved around the ownership of many of the queues. I changed it so that a task owns a queue if it is the one who sends data on the queue. That way when the task is shutdown, its only shutting down queues that it sends data on. I also made the shutdown functions owned by the tasks themselves and I made them blocking so that only one shutdown can occur at a time, that way I can choose the order that they are shutdown in. I noticed this caused a huge probelm with the web and ftp server because of the amount of time it would take to shutdown all tasks, so I created a special thread whose only purpose it to kill other tasks, after which it kills itself. This fixed the performance issue.

----

The main update here is the web interface. More error validation was added to prevent a gateway address and ip address from being on a different subnet. The application code was also updated to close the setpoint monitor, the btn driver and the other close functions were updated to free memory their respective tasks were using. The display driver was updated as well. The organization of certain elements were changed. An alarming bug was discovered that doesn't appear to be related to software. A seg module was saving its gpio incorrectly and this was causing a system crash. Its really bad because it appears to be a damaged memory cell, not something amendable in software. I just changed how it was saved and then it would read accurately again, assuming because it was using a different memory cell.

----

Added the ability to display the current temperature and setpoint in either celsius or fahrenheit. Toggling the UP button will switch between these.

----

Updated setpoint handler to fix bugs and be more responsive.

----

Optimized memory and cleaned up unused headers.

----

Basic setpoint handling has been added! There are more features to add with that but the basics appear to be working. Some refactoring was done as well.

----

Display handling has been added. Button handling added as well with software debouncing. The setpoint handler is started as well. Currently it only loads the current setpoint and saves it when there's a new one. Big change was made with the flash.ld file. RAM was optimized significantly, giving the application an additional 12 thousand bytes to works with! The change in init.c may not be necessary but it was added to prevent PA9 from being initialized twice.

----

----

Added a fix for data streaming from the http and ftp servers. The network interface has a limit to the number of bytes it can process and how quickly it can process. I modified a the tcp send and a function that returns buffer space available for transmit. I put a limit on the amount of bytes that can be streamed (no more than 1024) and I added a delay to tcp send so that any module that tries to stream will do so successfully, given the constraints with the network driver.

----

These files were removed from their directory and moved elsewhere. Forgot to include this in previous commit.

----

TCP directories were organized in a more logical way, a preliminary nand flash driver is included. It is not validated but could work someday if necessary. Support for an SD card was added. FTP and web server is running but can only handle small files. Some bug is present that prevents transmission of large packets. Probably an issue with NetworkInterface since it was never tested with large data streams.

----

Added base for display driver. It currently only supports 1 seg module and it counts from 0 to 9 on that module.

----

Updated the temperature monitor to use only 1 register instead of 4. It's not fully validated since the appropriate hardware is not available, but it's probably good. I also did a major cleanup on the project, organizing things. I removed the UDP logging task files since the task requires too much RAM to run. I made a small change to the network interface to prevent it from rebooting the chip before its had a chance to try connecting. The modbus task was updated to handle a single register write instead of 4, so it is simpler now.

----

Updated TempReadingMain.c to only show the whole number value of the temperature. Negative numbers are already in 2s complement so the only logic needed was grabbing the bits and extending the sign bit. Positive numbers only need to be grabbed since there is no sign bit.

----

Added temperature reading code. It's not fully validated but it appears to be reading temperatures. Changes to the network interface were made to improve network connection and semaphore protect the SPI master.

----

Fixed issues with network interface. Appears to work flawlessly now!

----

Made some changes to network interface to increase the reliability of the connection.

----

Forgot to add a couple files. This should be it for the addition of modbus to the project.

----

More deleted files...

----

These files need to be deleted.

----

Added support for modbus application protocol. A simulated temperature reading is available for reading. The network interface was improved slightly.

----

First commit! FreeRTOS is installed and the TCP IP stack is fully connected to the ksz8851snl chip through a fully ported NetworkInterface. An echo server is running to verify functionality. ICMP ping replies are working. Gratuitous ARP appears to work as well. UDP functionality has not been fully tested. Enabling the UDP printf task causes the device to run out of RAM so it cannot be used.